- Identification of the substance /mixture and of the company/undertaking

Product identifier

Product name: POZZOLANA ® 190PCS Component B
Other names.: Abrasion and stain resistance water-based polyurethane clear sealer

- Relevant identified uses of the substance or mixture and uses advised against

Use: 	Hardener

Restrictions on use: 	Do – it – yourself Application, Medical applications

- Details of the supplier of the safety data sheet

# Supplier:

POZZOLANA

3/20 Bearing Rd, Seven Hills 2147, Sydney NSW, Australia

Tel: 02 9838 8312

# Data sheet issuing department:

Laboratory:    Tel.: +84 975228336

Email	: technical@lavastone.world

- Emergency telephone number:

- Hazards identification

Tel.: 0404 930 695

- GHS Classification.

Acute toxicity (Inhalation): 				Category 4

Skin sensitization: 					Category 1

Specific target organ toxicity – single exposure: 	Category 3 (Respiratory system)

- GHS Label Elements

Hazard pictograms:

Signal word:			Warning

Hazard statements:		May cause an allergic skin reaction.

Harmful if inhaled.

May cause respiratory irritation.

Precautionary statements: 	Prevention:

Wear protective gloves.

Avoid breathing dust, mist, gas, vapors or spray.

Contaminated work clothing must not be allowed out of the workplace.

Use only outdoors or in a well-ventilated area.

Response:

IP ON SKIN: Wash with plenty of soap and water.

IF INHALED: Remove person to fresh air and keep at rest in a position comfortable for breathing.

Call a doctor or emergency medical facility (i.e.911) if you fell unwell.

If skin irritation or rash occurs: Get medical attention.

Wash contaminated clothing before reuse.

Storage:

Store in a well-ventilated place. Keep container tightly closed. Store locked up.

Disposal:

Dispose of contents and container in accordance with existing federal, state, and local environmental control laws.

- Composition/information on ingredients

## 3.1. Hazardous Components

| Concentration | Components | CAS-No. |
| --- | --- | --- |
| 60 – 80% | Homopolymer of Hexamethylene Diisocyanate | 28182-81-2 |
| 10 – 30% | Hydrophilic Aliphatic Polyisocyanate based on Hexamethylene Diisocyanate. | 666723-27-9 |
| <0.5% | Hexamethylene – 1,6 - Diisocyanate | 822-06-0 |

The specific chemical identity and/or exact percentage of component(s) have been withheld as a trade secret.

## 3.2. Other ingredients

Concentration			Components				CAS-No,

0.1 – 1% 			N,N – dimethylcyclohexylamine		98-94-2

This product contains an amine neutralizing agent which is bound in the matrix of this product as a salt. This amine salt is considered essentially unreactive at room temperature. Generation of anime vapors is expected when this product is processed (heated) during the drying/hardening of the coating.

- First aid measures

## Most Important Symptom(s)/Effect(s)

## Acute: Isocyanate vapors or mist at concentrations above the exposure limits or guidelines can irritate (burning sensation) the mucous membranes in the respiratory tract (nose, throat, lungs) with symptoms of runny nose, sore throat, coughing, chest discomfort, shortness of breath and reduced lung function (breathing difficulty). Persons with a preexisting, nonspecific bronchial hyper reactivity can respond to concentration below the exposure limits or guidelines with similar symptoms as well as asthma attack or asthma-like symptoms. Exposure well above the exposure limits or guidelines may lead to bronchitis, bronchial spasm and pulmonary edema (fluid in lungs). Chemical or hypersensitivity pneumonitis, with flu-like symptoms (e.g. fever. Chills), has also been reported. These symptoms can be delayed up to several hours after exposure. These effects are usually reversible.

May cause skin irritation with symptoms of reddening, itching, and swelling. Can cause sensitization. Persons previously sensitized can experience allergic skin reaction with symptoms of reddening, itching, swelling, and rash. Cured material is difficult to remove.

May cause eye irritation with symptoms of reddening, tearing, stinging, and swelling. May cause temporary corneal injury. Vapor or aerosol may cause irritation with symptoms of burning and tearing.

May cause irritation of the digestive tract; Symptoms may include abdominal pain, nausea, vomiting, and diarrhea.

Delayed: Symptoms affecting the respiratory tract can also occur several hours after overexposure.

Eye Contact

In cause of contact, immediately flush eyes with plenty of water for at least 15 minutes. Use lukewarm water if possible. Use fingers to ensure that eyelids are separated and that the eye is being irrigated. Then remove contact lenses, if easily removable, and continue eye irrigation for not less than 15 minutes. Get medical attention of irritation develops.

Skin Contact

If direct skin contact with isocyanates occurs, immediately remove contaminated clothing and shoes. Wipe off the isocyanate product from the skin using dry towels or other similar absorbent fabric. If readily available, apply a polyglycol-based cleanser (e.g. SKC, Inc. (SKC) D-TAMTM Skin Cleanser) or corn oil. Wash with soap and warm for 15 minutes. If available, use a wipe test pad to verify decontamination is complete (e.g. SKC SWYPETM). Get medical attention if irritation develops. Discard or wash contaminated clothing before reuse.

Inhalation

Move to an area free from further exposure. Extreme asthmatic reactions that may occur in sensitized persons can be life threatening. Get medical attention immediately. Administer oxygen or artificial respiration as needed. Asthmatic symptoms may develop and may be immediate or delayed up to several hours.

Ingestion

DO NOT induce vomiting. Wash mouth out with water. Do not give anything by to an unconscious person. Get medical attention.

Notes to Physician

Eyes: Stain for evidence of corneal injury. If cornea is burned, instill antibiotic/steroid preparation as needed. Workplace vapors could produce reversible corneal epithelial edema impairing vision.

Skin: This compound is a skin sensitizer. Treat symptomatically as for contact dermatitis or thermal burn.

Ingestion: Treat symptomatically. There is no specific antidote. Inducing vomiting is contraindicated because of the irritating nature of the compound.

Inhalation: Treatment is essentially symptomatic. An individual having dermal or pulmonary sensitization reaction to this material should be removed from further exposure to any diisocyanate.

- Firefighting measures

## 5.1 Extinguishing media

Suitable extinguishing media: Carbon dioxide (CO2), Foam, extinguishing powder, in cases of larger fires, water spray should be used.

Unsuitable extinguishing media: High volume water jet

## 5.2 Special hazards arising from the substance or mixture

Burning releases carbon monoxide, carbon dioxide, oxides of nitrogen and traces of hydrogen cyanide. In the event of fire and/or explosion do not breathe fumes.

## 5.3 Advice for fire-fighters

Firemen must wear self-contained breathing apparatus.

- Accidental release measures

Do not allow contaminated extinguishing water to enter the soil, ground-water or surface waters.

  - Personal precautions, protective equipment and emergency procedures

Put on protective equipment (see section B). Ensure adequate ventilation/exhaust extraction, Keep unauthorized persons away.

  - Environment related measures

Do not allow to escape into waterways, wastewater or soil.

  - Methods and material for containment and cleaning up: Take up with absorbent for chemicals or, if necessary with dry sand and store in closed containers.

  - Reference to other sections

- Handling and storage

For further disposal measures see section 13.

## 7.1 Precautions for safe handling

When handling observe the usual precautionary measures for chemicals. Avoid contact with the skin and the eyes.

Keep away from foodstuffs, drinks and tobacco. Wash hands before breaks and at the end of workday. Keep working clothes separately. Change contaminated or soaked clothing.

## 7.2 Conditions for safe storage, including any incompatibilities

Keep container tightly closed in a dry and well-ventilated place. Further information on the storage conditions which must be observed to preserve quality can be found in our product information sheet.

Storage class (TRGS 510): 	12: Non Combustible Liquids

7.3 Specific end use(s)

No information available.

- Exposure controls/personal protection

  - Control parameters: None

No information on Exposure Limit Values necessary according to EC directive 2006/121/EG

The neutralizing agent is released during processing.

| Substance | CAS-No. | Basis | Type | Value | Ceiling Limit Value | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| anhydrous ammonia | 7664-41-7 | EU ELV | TWA | 20 ppm 14 mg/m3 |  | Indicative |
| anhydrous ammonia | 7664-41-7 | EU ELV | STEL | 50 ppm 36 mg/m3 |  | Indicative |
| anhydrous ammonia | 7664-41-7 | TRGS 900 |  |  |  | Listed. |
| anhydrous ammonia | 7664-41-7 | TRGS 900 |  | 20 ppm 14 mg/m3 | 2 | Y |
| anhydrous ammonia | 7664-41-7 | TRGS 900 | STEL CL |  |  | Category I: substances for which the localized effect has an assigned OEL or for substances with a sensitizing effect in respiratory passages. |

# 8.2. Exposure controls

Respiratory protection

Respiratory protection required in insufficiently ventilated working areas and during spraying.

## Hand protection

Suitable materials for safety gloves; EN 374:

Fluorinated rubber - FKM: thickness >=0,4mm; breakthrough time >=480min.

Butyl rubber - IIR: thickness >=0,5mm; breakthrough time >=480min.

Nitrile rubber - NBR: thickness >=0,35mm; breakthrough time >=480min. Recommendation: contaminated gloves should be disposed of.

Eye protection

Wear eye/face protection.

Skin and body protection

Wear suitable protective clothing.

# Appearance

  - Form: Liquid

  - Color: Milk white

Odor: Odorless

Change of state

- Melting point: Inapplicable

- Flashpoint: Inapplicable

Explosive properties: The product is not self-igniting.

Density (25°C): 1.0 kg/dm3

# Viscosity dynamic: 10 – 50 MPa.s

pH-value: 3.5%

- Physical and chemical properties

Organic solvents: 0.0%

## 9.1 Information on basic physical and chemical properties

| Appearance: | liquid, thixotropic |  |  |  |
| --- | --- | --- | --- | --- |
| Color: | Milk white |  |  |  |
| Odor: | slight inherent odor |  |  |  |
| Odor Threshold: | not established |  |  |  |
| Freezing temperature: | ca. 0 °C |  | ISO 3016 |  |
| Boiling point/boiling range: | ca. 100 °C at 1.004 hPa |  | EG A2 |  |
| Flash point: | No flash point up to initial boiling point. |  | DIN EN ISO |  |
|  |  |  | 2719 |  |
| Evaporation rate: | not established |  |  |  |
| Flammability (solid, gas): | not applicable |  |  |  |
| Burning number: | not applicable |  |  |  |
| Vapor pressure: | ca. 126 hPa at 55 °C |  | EG A4 |  |
|  | ca. 124 hPa at 50 °C |  | EG A4 |  |
| Vapor density: | not established |  |  |  |
| Density: | ca. 1,04 g/cm³ at 20 °C |  | DIN 51757 |  |
| Miscibility with water: | miscible at 15 °C |  |  |  |
| Surface tension: | not established |  |  |  |
| Partition coefficient (n−octanol/water): | not established |  |  |  |
| Auto-ignition temperature: | not applicable |  |  |  |
| Ignition temperature: | ca. 440 °C at 997 hPa |  |  | DIN 51794 |
| Decomposition temperature: | not established |  |  |  |
| Viscosity, dynamic: | ca. 26 mPa.s at 20 °C |  |  | DIN 53019 |
| Explosive properties: | not established |  |  |  |
| Dust explosion class: | not applicable |  |  |  |
| Oxidizing properties: | not established |  |  |  |

## 9.2 Other information

- Stability and reactivity

The indicated values do not necessarily correspond to the product specification. Please refer to the technical information sheet for specification data.

10.1 Reactivity

This information is not available.

10.2 Chemical stability

No thermal decomposition when stored and handled correctly.

10.3 Hazardous decomposition products

- Toxicological information

On drying of the coating / hardening release of neutralizing agent. (see section 3)

Toxicological studies on the product are not yet available.

Please find below the toxicological data available to us for the components.

## 11.1 Information on toxicological effects acute toxicity, oral

Polyacrylate dispersion

LD50 rat: > 5.000 mg/kg

Toxicological studies of a comparable product.

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1) Assessment: Toxic if swallowed.

Suppliers' information

## Acute toxicity, dermal

Polyacrylate dispersion

Assessment: The substance or mixture has no acute dermal toxicity Studies of a comparable product.  5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1) Assessment: Fatal in contact with skin (Category 2).

Suppliers' information

## Acute toxicity, inhalation

Polyacrylate dispersion

Assessment: The substance or mixture has no acute inhalation toxicity Studies of a comparable product.

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1)

LC50 rat:  0,31 mg/l, 4 h

Test atmosphere: dust/mist

Assessment: Fatal if inhaled (Category 2).

Classification (1272/2008/CE)

## Primary skin irritation

Polyacrylate dispersion

Species: rabbit

Result: non-irritant

Classification: No skin irritation

Toxicological studies of a comparable product.

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1)

Result: Corrosive

Classification: Causes severe skin burns and eye damage (Skin Corr. 1B).

## Primary mucosae irritation

Polyacrylate dispersion

Species: rabbit

Result: slight irritant

Classification: No eye irritation

Toxicological studies of a comparable product.

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1)

Since this substance is already classified “corrosive”, the risk of serious damage to the eyes is implicit.

## Sensitization

Polyacrylate dispersion

Skin sensitization (local lymph node assay (LLNA)):

Species: Mouse

Result: negative

Classification: Does not cause skin sensitization.

Method: OECD Test Guideline 429

Toxicological studies of a comparable product.

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1) Skin sensitization:

Species: Guinea pig

Result: positive

Classification: May cause sensitization by skin contact.

## Subacute, sub chronic and prolonged toxicity

Polyacrylate dispersion No data available.

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1) No data available.

## Carcinogenicity

Polyacrylate dispersion No data available.

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1) Animal testing did not show any carcinogenic effects.

## Reproductive toxicity/Fertility

Polyacrylate dispersion No data available.

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1) Available data show no indications for reproductive toxicity.

## Reproductive toxicity/Teratogenicity

Polyacrylate dispersion No data available.

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1) Based on available data, the classification criteria are not met.

## Genotoxicity in vitro

Polyacrylate dispersion

Test type: Salmonella/microsome test (Ames test) Result: No indication of mutagenic effects.

Method: OECD Test Guideline 471

Toxicological studies of a comparable product.

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1) Result: No indication of mutagenic effects.

Suppliers' information

Genotoxicity in vivo Polyacrylate dispersion No data available.

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1) No data available.

STOT evaluation – one-time exposure Polyacrylate dispersion No data available.

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1) No data available.

STOT evaluation – repeated exposure Polyacrylate dispersion No data available.

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1) No data available.

## Aspiration toxicity

Polyacrylate dispersion No data available.

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1) No data available.

## CMR Assessment

Polyacrylate dispersion

Carcinogenicity: No data available.

Mutagenicity: Based on available data, the classification criteria are not met.

Teratogenicity: No data available.

Reproductive toxicity/Fertility: No data available.

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1) Carcinogenicity: Based on available data, the classification criteria are not met.

Mutagenicity: Based on available data, the classification criteria are not met.

Teratogenicity: Based on available data, the classification criteria are not met.

- Ecological Information

Reproductive toxicity/Fertility: Based on available data, the classification criteria are not met.

Ecotoxicological studies of the product are not available.

Do not allow to escape into waterways, wastewater or soil.

Please find below the ecotoxicological data available to us for the components.

## 12.1 Toxicity Acute Fish toxicity

Polyacrylate dispersion

LC50 > 100 mg/l

Species: Danio rerio (zebra fish)

Exposure duration: 96 h

Method: OECD Test Guideline 203

Ecotoxicological reports on a comparable product

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1)

LC50 0,19 mg/l

Species: Oncorhynchus mykiss (rainbow trout)

Exposure duration: 96 h

Information refers to the main component.

Chronic Fish toxicity Polyacrylate dispersion No data available.

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1)

NOEC 0,05 mg/l

Species: Oncorhynchus mykiss (rainbow trout)

Exposure duration: 14 d

## Acute toxicity for daphnia

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1)

EC50 0.16 mg/l

Species: Daphnia magna (Water flea)

Exposure duration: 48 h

Information refers to the main component.

Chronic toxicity to daphnia Polyacrylate dispersion No data available.

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1)

NOEC 0,1 mg/l

Species: Daphnia magna (Water flea)

Exposure duration: 21 d

Acute toxicity for algae Polyacrylate dispersion No data available.

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1)

EC50 27 µg/l

Species: Pseudokirchneriella subcapitata (green algae)

Exposure duration: 72 h

Method: OECD Test Guideline 201

## Acute bacterial toxicity Polyacrylate dispersion

EC50 > 10.000 mg/l

Species: activated sludge

Method: OECD Test Guideline 209

Ecotoxicological reports on a comparable product

## Ecotoxicology Assessment

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1) Acute aquatic toxicity: Very toxic to aquatic life.

Chronic aquatic toxicity: Very toxic to aquatic life with long lasting effects.

## M-Factor

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1)

M-factor (acute aquat. tox.): 100

M-factor (chron. aquat. tox.): 100

## 12.2 Persistence and degradability Biodegradability

Polyacrylate dispersion

Biodegradation: 5 %, 28 d, i.e. not readily degradable

Method: OECD Test Guideline 301 D

Ecotoxicological reports on a comparable product

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1) Biodegradation: < 50 %, 10 d, i.e. not readily degradable

## 12.3 Bio accumulative potential Partition coefficient (n−octanol/water)

5-chloro-2-methyl-3(2H)-isothiazolone / 2-methyl-3(2H)-isothiazolone (3:1)

log Pow:  0,401

12.4 Mobility in soil No data available.

## 12.5 Results of PBT and vPvB assessment

This substance/mixture contains no components considered to be either persistent, bioaccumulative and toxic (PBT), or very persistent and very bio accumulative (vPvB) at levels of 0.1% or higher.

12.6 Other adverse effects No data available.

- Disposal considerations

Dispose in accordance with applicable international, national and local laws, ordinances and statutes.

For disposal within the EC, the appropriate code according to the European Waste Catalogue (EWC) should be used.

## 13.1 Waste treatment methods

After containers have been emptied as thoroughly as possible (e.g. by pouring, scraping or draining until "drip-dry"), they can be sent to an appropriate collection point set up within the framework of the existing take-back scheme of the chemical industry. Containers must be recycled in compliance with national legislation and environmental regulations.

- Transport informations

None disposal into waste water.

## ADR/RID

14.1 UN number 	: 	Not dangerous goods

14.2 UN proper shipping name 	: 	Not dangerous goods

14.3 Transport hazard class(es) 	: 	Not dangerous goods 14.4 Packing group 	: 	Not dangerous goods

14.5 Environmental hazards 	: 	Not dangerous goods

## ADN

14.1 UN number 	: 	Not dangerous goods

14.2 UN proper shipping name 	: 	Not dangerous goods

14.3 Transport hazard class(es) 	: 	Not dangerous goods

14.4 Packing group 	: 	Not dangerous goods

14.5 Environmental hazards 	: 	Not dangerous goods

Dangerous goods classification for inland waterways tanker by request only.

| 14.5 Marine pollutant : 14.6 Special precautions for user See section 6 - 8. | Not dangerous goods |
| --- | --- |
| Additional information : | Not dangerous cargo. Avoid temperatures below 0 °C. Keep away from foodstuffs, acids and alkalis. |

14.7 Transport in bulk according to Annex II of MARPOL73/78 and the IBC Code

- Regulatory informations

Not applicable.

## 15.1 Safety, health and environmental regulations/legislation specific for the substance or mixture

Directive 2012/18/EU on the control of major-accident hazards involving dangerous substances.

not applicable

## TA Luft List (Germany)

Type: Total dust

Fraction of other substances: 0 %

Type: Organic Substances

Fraction of other substances: 42 %

## Water contaminating class (Germany)

1 	slightly water endangering

Classification according to AwSV, Annex 1 (5.2)

15.2 Chemical Safety Assessment

- Other informations

A Chemical Safety Assessment has not been conducted for this substance / mixture resp. its components.

Full text of the hazard statements of the CLP classification (1272/2008/CE) referred to under sections 2, 3 and 10.

| H221 | Flammable gas. |
| --- | --- |
| H301 | Toxic if swallowed. |
| H302 | Harmful if swallowed. |
| H310 | Fatal in contact with skin. |
| H314 | Causes severe skin burns and eye damage. |
| H317 | May cause an allergic skin reaction. |
| H318 | Causes serious eye damage. |
| H330 | Fatal if inhaled. |
| H331 | Toxic if inhaled. |
| H400 | Very toxic to aquatic life. |
| H410 | Very toxic to aquatic life with long lasting effects. |

## Abbreviations and acronyms

| ADN | Accord européen relatif au transport international des marchandises Dangereuses par voie de Navigation intérieure |
| --- | --- |
| ADR | Accord européen relatif au transport international des marchandises Dangereuses par Route |
| ANSI | American National Standards Institute |
| ASTM | American Society of Testing and Materials (US) |
| ATE | Acute Toxic Estimate |
| AwSv | Verordnung über Anlagen zum Umgang mit wassergefährdenden Stoffen |
| BCF | Bioconcentration Factor |
| CAS | Chemical Abstract Service |
| CLP | Regulation on Classification, Labelling and Packaging of Substances and Mixtures |
| CMR | Cancerogenic Mutagenic Reprotoxic |
| DIN | Deutsches Institut für Normung |
| DNEL | Derived No-Effect Level |
| EC… | Effect Concentration ... % |
| EWC | European Waste Catalogue |
| IATA | International Air Transport Association |
| IBC | Intermediate Bulk Container |
| ICAO | International Civil Aviation Organization |
| IMDG | International Maritime Dangerous Goods |
| IMO | International Maritime Organization |
| ISO | International Organization for Standardization |
| IUPAC | International Union of Pure and Applied Chemistry |
| LOAEL | Lowest Observable Adverse Effect Level |
| LC… | Lethal Concentration, ...% |
| LD… | Lethal Dose, ...% |
| MARPOL | International Convention for the Prevention of Pollution From Ships |
| NOAEL | No Observed Adverse Effect Level |
| NOEL/NOEC | No Observed Effect Level/Concentration |
| OECD | Organisation for Economic Co-operation and Development |
| PBT | persistent, bioaccumulative, toxic |
| PNEC | Predicted No-Effect Concentration |
| REACH | Registration, Evaluation, Authorisation and Restriction of Chemicals |
| RID | Règlement concernant le transport International ferroviaire de marchandises Dangereuses |
| STOT | Specific Target Organ Toxicity |
| TRGS | Technische Regeln für Gefahrstoffe |
| vPvB | very Persistent, very Bioaccumulative |
| WGK | Wassergefährdungsklasse |

Changes since the last version are highlighted in the margin. This version replaces all previous versions.

## Further information

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process, unless specified in the text.