- Identification of the substance /mixture and of the company/undertaking

Product identifier

Product name: POZZOLANA ® 730 SERIES
Other Names: Premium Mineral Silicate binder for wall surface plastering and rendering.

- Relevant identified uses of the substance or mixture and uses advised against

No additional information available.

Relevant identified uses of the substance or mixture: Multi-purpose, rendering product for commercial decorative wall.

- Details of the supplier of the safety data sheet

# Supplier:

POZZOLANA

3/20 Bearing Rd, Seven Hills 2147, Sydney NSW, Australia

Tel: 02 9838 8312

# Data sheet issuing department:

Laboratory:    Tel.: +84 975228336

Email	: technical@lavastone.world

- Emergency telephone number:

- Hazards identification

Tel.: 0404 930 695

- Classified as Hazardous according to the Globally Harmonized System (GHS).

# GHS Classification:

- Carcinogenicity, Category 1A (H350) – Respirable crystalline silica

- STOT (Repeated Exposure), Category 1 (H372) – Respirable crystalline silica

- Serious Eye Damage, Category 1 (H318) – Cement (alkaline)

- Skin Irritation, Category 2 (H315) – Cement, alkaline reaction

- Skin Sensitisation, Category 1 (H317) – Traces of Chromium (VI) in Portland cement

- STOT (Single Exposure), Category 3 (H335) – Cement dust (respiratory irritation)

- Hazard Statements

- H350: May cause cancer (inhalation).

- H372: Causes damage to organs (lungs) through prolonged or repeated exposure (inhalation of respirable crystalline silica).

- H318: Causes serious eye damage.

- H315: Causes skin irritation.

- H317: May cause an allergic skin reaction.

- H335: May cause respiratory irritation.

- Precautionary Statements

- Prevention

- P201: Obtain special instructions before use.

- P202: Do not handle until all safety precautions have been read and understood.

- P260: Do not breathe dust.

- P280: Wear protective gloves/protective clothing/eye protection/face protection.

- P284: In case of inadequate ventilation, wear respiratory protection.

- Response

- P302 + P352: IF ON SKIN: Wash with plenty of water.

- P305 + P351 + P338: IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses if present and easy to do. Continue rinsing.

- P310: Immediately call a POISON CENTER/doctor.

- P308 + P313: IF exposed or concerned: Get medical advice/attention.

- P333 + P313: If skin irritation or rash occurs: Get medical advice/attention.

- Disposal

P501: Dispose of contents/container in accordance with local/regional/national regulations.

- Composition/information on ingredients

All percentages are approximate and may vary by product formulation.

| INGREDIENTS | Approx. % (w/w) | GHS Classification |
| --- | --- | --- |
| Aluminous Cement | Less than. 10 % | Can cause skin/eye irritation (refer to analysis for exact category) |
| Supplementary Cementitious Materials | Less than. 50 % (combined) | Generally, not classified as hazardous; dust precautions recommended |
| Limestone | Less than. 30 % | Not classified; dust may irritate (mechanical) |

Note on SCMs:

- First aid measures

This mixture may include Ground Granulated Blast Furnace Slag (GGBS; CAS 65996‐69‐2), Fly Ash (CAS 68131‐74‐8), and/or Silica Fume (CAS 69012‐64‐2). None are classified as hazardous if they do not contain significant respirable crystalline silica or other heavy metals. The combined SCMs are grouped together, as they do not individually exceed classification thresholds.

  - Inhalation:

- Remove to fresh air.

- Seek medical attention if symptoms persist (e.g., coughing, difficulty breathing).

  - Skin Contact:

- Wash thoroughly with soap and water.

- Remove contaminated clothing.

- If irritation or rash develops, seek medical advice.

  - Eye Contact:

- Rinse with water for at least 15 minutes.

- Remove contact lenses if present and easy to do.

- Seek immediate medical attention due to risk of serious eye damage.

  - Ingestion:

- Rinse mouth; do not induce vomiting unless directed by medical personnel.

- If unwell, seek medical advice.

Symptoms / Effects:

- Immediate: Irritation to eyes, skin, respiratory system (alkaline cement, dust).

- Delayed: Possible allergic dermatitis (Cr [VI] in cement), silicosis (chronic inhalation).

- Firefighting measures

Extinguishing Media: Non‐combustible. Use media suitable for surrounding fire (e.g., water spray, CO₂, foam).

- Fire/Explosion Hazards: None inherent to the product itself.

- Advice for Firefighters: Use self‐contained breathing apparatus if exposed to dust or decomposition products in confined areas.

- Accidental release measures

  - Personal precautions, protective equipment and emergency procedures

Avoid creating dust.

Use appropriate PPE (respirator, gloves, goggles).

  - Environmental precautions

Prevent large spills from entering drains or waterways.

  - Clean-Up Methods:

Sweep or vacuum gently without generating dust.

Collect in labeled containers for reuse or disposal.

- Handling and storage

- Safe Handing:

Handle in well-ventilated area.

Avoid breathing dust and contact with skin/eyes.

Wear protective equipment per section 8.

- Storage

Store in original containers, sealed and dry.

Keep away from moisture and incompatible materials (e.g., strong acids).

- Exposure controls/personal protection

  - Exposure Standards (Safe Work Australia)

# Personal protective equipment:

General protective and hygienic measures:

Keep away from food, beverages and animal feed. Change soaked clothes.

Do not inhale dust / smoke / fog.

Avoid contact with eyes and skin.

Do not eat, drink, smoke or sniff during work. Clean skin after work and before pauses.

Use regreasing skin protection cream.

  - Engineering Controls

- Use local exhaust or general ventilation to keep dust concentrations below exposure limits.

- Provide eyewash and washing facilities.

# 8.3. Personal protective equipment (PPE)

# Respiratory protection:

Dust mask

P2 or P3 dust mask/respirator (AS/NZS 1715 & 1716) if airborne dust is not controlled below limits.

Eye protection:

Safety glasses

Goggles or face shield (AS/NZS 1337)

Skin protection:

Protective clothing

Protection gloves

Wear gloves (e.g., nitrile), long sleeves, and safety boots.

Hygiene Measures:

- Physical and chemical properties

Wash hands and exposed skin thoroughly after handling; do not eat/drink/smoke in work areas.

Property			Value/Description

Appearance			White

Odour				Slight or no odour

Flash Point			Not applicable (non‐flammable)

Flammability			Not flammable

Vapour Pressure		Not applicable (solid)

Bulk Density			~ 1.0 g/cm³

Solubility (Water)		Slight; reacts to form hydrated compounds

Auto‐ignition Temperature	Not applicable

- Stability and reactivity

Decomposition Temperature	Not established

  - Reactivity: Alkaline reaction upon mixing with water.

  - Chemical stability: Stable if kept dry.

- Hazardous reactions: Contact with strong acid may release CO2 (limestone) and heat (cement).

- Conditions to Avoid: Moist or humid storage, which can cause partial hydration.

- Incompatible Materials: Acids, ammonium salts.

- Hazardous decomposition products: None under normal conditions.

- Toxicological information

# Acute Toxicity

- Overexposure to dust may irritate respiratory tract, eyes, skin.

# Skin Corrosion/Irritation

- Alkaline cement can cause irritation or burns if combined with moisture.

# Serious Eye Damage/Irritation

- Cement dust or wet mix can cause serious eye damage.

# Respiratory or Skin Sensitisation

- Trace Cr(VI) in cement can cause skin sensitisation.

# STOT (Single Exposure)

- Dust inhalation may lead to respiratory irritation (H335).

# STOT (Repeated Exposure)

- Chronic inhalation of respirable crystalline silica may lead to silicosis, lung cancer (H372).

# Carcinogenicity

- Ecological information

Respirable crystalline silica is a Category 1A carcinogen (IARC) (H350).

- Ecotoxicity: High pH of the wet mixture can be harmful to aquatic life if large amounts enter waterways.

- Persistence/Degradability: Once hardened, product becomes largely inert.

- Bioaccumulative Potential: None expected.

- Disposal considerations

Mobility in Soil: Dust may become airborne; hardened product has low mobility.

# Waste Disposal:

- Dispose of in compliance with local regulations.

- Unused powder or wet mix: avoid discharge into drains.

- Hardened product is typically inert; dispose of as construction waste if uncontaminated.

# Packaging Disposal:

- Empty bags may still contain residual dust; handle accordingly.

- Transportation Information

- UN Number:		 Not applicable (non-dangerous goods).

- Transport Hazard Class:	 Not applicable.

- Proper Shipping Name: 	 Not applicable.

- Packing Group: 		 Not applicable.

- Regulatory Information

Environmental Hazards: 	 Not classified as marine pollutant or dangerous for transport.

- Safety, health and environmental regulations/legislation specific for the substance or mixture

National regulations:

Classification according to industrial safety regulations (BetrSichV):

- Water hazard class: WGK 1 (self-classification): Slightly hazardous to water.

Additional regulations, limitations and prohibitive regulations ：

- BGR 190 ''Rules for the use of respiratory protective devices''

- BGR 192 "Rules for the use of equipment for eye and face protection" BGR 189 "Rules for the use of protective clothing "

- BGR 195 "Rules for the use of protective gloves "

- Chemical Safety Assessment: For this substance a chemical safety assessment is not required.

- Any other relevant information

References/ Key Legend:

- GHS: Globally Harmonized System

- STOT: Specific Target Organ Toxicity

- RE: Repeated Exposure

- SE: Single Exposure

- TWA: Time‐Weighted Average

- SCMs: Supplementary Cementitious Materials

Disclaimer:

This information is based on our present state of knowledge. However, it should not constitute a guarantee for any specific product properties and shall not establish a legally valid relationship.